Table 1: Board Setting Configuration [SW6]

| Switch | Schematic Signal Name | Description | Settings |
| --- | --- | --- | --- |
| 1 | CLK_SEL | ON for 100Mhz on-board clock oscillator selection OFF for SMA input clock selection | ON |
| 2 | CLK_EN | OFF for setting CLK_ENABLE signal high to the MAXV | OFF |
| 3 | Si516_FS | ON for setting the SDI REFCLK frequency to 148.35Mhz OFF for setting the SDI REFCLK frequency to 148.5Mhz | Depends on the source of video to SDI II Receiver (Channel 0),ON for NTSC,OFF for PAL |
| 4 | FACTORY | ON to load user design from flash OFF to load factory design from flash | ON |
| 5 | ZQ_B2K | ON for setting RZQ resistor of Bank 2K to 99.17ohm OFF for setting RZQ resistor of Bank 2K to 240ohm | OFF |

Table 2: DIP Switches

| DIP Switch | SW2 |
| --- | --- |
| 8 | 0 = 75% colorbars 1 = 100% colorbars |
| 7 | 0 = output colorbars 1 = output pathological test pattern |
| 6 | 0 = output color 1 = output no color |
| 5 | 0 = PAL rate 1 = NTSC rate |
| 4:1 | 4’b0000: SD – 525i 4’b0001: SD – 625i 4’b0010: HD – 1080i60/59.94 4’b0011: HD – 1080i50 4’b0100: HD – 1080p24/23.96 4’b0101: HD – 720p60/59.94 4’b0110: HD – 720p30/29.97 4’b0111: HD – 1080p30/29.97 4’b1000: HD – 1080p25 4’b1001: 3Ga – 1080p60/59.94 4’b1010: 3Ga – 1080p50 4’b1011: 3Gb – 2x1080p60 4’b1100: 3Gb – 2x720p30/29.97 4’b1101: 3Gb – 2x1080p60/59.94 4’b1110: 3Gb – 1080p60/59.94 4’b1111: 3Gb – 1080p50 |

Table 6: User LEDs

| User LEDs | Description |
| --- | --- |
| D3 | The heartbeat of the transmitter clock out for channel 0 |
| D4 | The heartbeat of the receiver recovered clock out for channel 0 |
| D5 | Frame locked for channel 0 |
| D6 | TRS locked for channel 0 |
| D7,D8 | RX signal standard for channel 0: - SD :[D7,D8] = 00 - HD :[D7,D8] = 01 - 3Ga :[D7,D8] = 11 - 3Gb :[D7,D8] = 10 |
| D9,D10 | Internal pattern generator signal standard for channel 1: - SD :[D9,D10] = 00 - HD :[D9,D10] = 01 - 3Ga :[D9,D10] = 11 - 3Gb :[D9,D10] = 10 |